# Marine Propeller Shaft Design Manual

## A Complete Parametric Design Guide using Solid Edge

Project: Design of a Hollow Propeller Shaft for a 300 kW Marine Vessel

Material: Duplex Stainless Steel 2205

Key Feature: Parametric Modeling with Tapered Spline Connection

## Table of Contents

- Introduction & Scope

- Design Philosophy & Industry Standards

- Specifications & Material Selection

- Parametric Modeling Strategy (The Core Logic)

- Step-by-Step CAD Modeling Guide

  - 5.1. Variable Table Setup

  - 5.2. The Revolve Profile (Main Shaft Body)

  - 5.3. The Tapered Spline (Lofted Cutout Method)

  - 5.4. The Flange Connection

  - 5.5. Simulation Preparation (Split Faces)

  - 5.6. Final Detailing (Fillets)

- Manufacturing & Assembly Considerations

- Conclusion

- Appendix: The Master Variable Table

## 1. Introduction & Scope

This document serves as a comprehensive guide to designing a Marine Propeller Shaft. It is important to distinguish this component:

- Propeller Shaft: The aft-most section of the shaft line that connects directly to the propeller. It includes a taper, splines (or keyway), and a thread for the locking nut.

- Propulsion Shaft (Intermediate Shaft): Connects the propeller shaft to the gearbox/engine via flanges.

Our design focuses on the Propeller Shaft, utilizing a hollow cross-section and involute tapered splines for superior performance.

## 2. Design Philosophy & Industry Standards

Why did we choose this specific design over a traditional solid shaft?

### 2.1 Hollow vs. Solid Shaft

- Traditional: Solid shafts are heavy, and the center material carries zero torque (stress is zero at the neutral axis).

- Our Design: A Hollow Shaft removes this "dead weight," significantly improving the strength-to-weight ratio. This is a standard in high-performance naval architecture and racing vessels.

### 2.2 Splines vs. Keyways

- Traditional: A single keyway is cheap to manufacture but creates a massive stress concentration point, leading to fatigue cracks.

- Our Design: Tapered Splines distribute the torque load equally across 6 teeth. This is a "Premium" feature found in aerospace and high-end marine applications, offering superior fatigue life.

### 2.3 Straight Thread vs. Tapered Thread

- Our Choice: A Straight Cylindrical Thread (R50).

- Reason: Standard locking nuts are designed for straight threads. Tapered threads are impractical for axial clamping in this application.

## 3. Specifications & Material Selection

- Engine Power: 298 kW (CAT 3406C)

- Shaft Speed: 225 RPM (after 8:1 reduction)

- Material: Duplex Stainless Steel 2205

  - Advantage: Higher yield strength (460 MPa) and superior pitting corrosion resistance compared to standard 316L steel. Eliminates the need for protective liners.

- Dimensions:

  - Length: 1500 mm

  - OD: 125 mm | ID: 60 mm

  - Taper Length: 300 mm (1:16 Taper)

## 4. Parametric Modeling Strategy (The Core Logic)

To make this design "Industry Ready," we use Parametric Modeling. We do not hard-code dimensions. Instead, we use a central Variable Table.

Key Formulas Used:

- Spline Root Clearance: To ensure the propeller slides over the thread without hitting it.R_Thread = R_SmallEnd - Spline_Depth - Clearance (1mm)

- Taper Angle Calculation: Automatically calculates the angle for the spline draft.Shaft_Taper_Angle = atn((OR_Shaft - R_SmallEnd) / L_Taper) * (180/PI)

- Bearing Width: Scaling the bearing seat based on shaft diameter.Bearing_Width = OR_Shaft * 2 + Tolerance

## 5. Step-by-Step CAD Modeling Guide

This workflow is optimized for Solid Edge, ensuring no errors during parametric updates.

### 5.1. Variable Table Setup

- Before drawing anything, open Tools > Variables.

- Enter the base values (e.g., L_Total = 1500, OR_Shaft = 62.5).

- Define formulas as shown in the Appendix.

### 5.2. The Revolve Profile (Main Shaft Body)

- Sketch (XZ Plane): Draw the upper half of the cross-section.

  - Thread Section: A straight horizontal line.

  - The Step: A vertical line rising from the thread to the taper start.

  - The Taper: A sloped line.

  - Shaft Body: Straight horizontal line.

  - Flange: Vertical rise at the end.

- Constraints: Ensure the thread section is strictly horizontal (Not tapered).

- Dimensions: Link all dimensions to your Variables (e.g., =L_Taper, =R_SmallEnd).

- Revolve: Create the 3D solid.

### 5.3. The Tapered Spline (Lofted Cutout Method)

Note: We use Lofted Cutout because Drafted Extrusion distorts the depth on conical surfaces.

- Sketch 1 (Start): Create a sketch on the R53 face (The Step). Draw the spline gap profile using Smallend variables.

- Sketch 2 (End): Create a Parallel Plane at distance =L_Taper. Draw the gap profile using Biggerend variables.

- Loft: Use Lofted Cutout to join Sketch 1 and Sketch 2. This creates a perfect constant-depth groove.

- The "Runout" Logic: Since our Thread cross section is lower than the Spline Root cross section, the Lofted Cutout naturally cuts through the thread-spline connected cross section "Step," creating an entry path for the propeller. No separate runout cut is strictly needed for geometry, but it happens naturally.

- Pattern: Circular Pattern the cutout (Count =6).

### 5.4. The Flange Connection

- Hole: Create a simple hole on the flange face.

- Position: Set the PCD (Pitch Circle Diameter) using the formula =OR_Flange * 2 - Clearance.

- Pattern: Circular Pattern the hole (Count =6).

### 5.5. Simulation Preparation (Split Faces)

Crucial for COMSOL: We do not model bearings. We "mark" their location.

- Sketch: Draw vertical lines intersecting the shaft axis on the XZ plane.

- Positioning: Use formulas (e.g., =Pos_Fwd_Bearing +/- Width/2) to keep them centered parametrically.

- Extrude Surface: Extrude these lines as surfaces through the shaft.

- Split Face: Use the surfaces to split the cylindrical face of the shaft.

- Result: Distinct "bands" on the shaft surface to apply bearing loads in simulation.

### 5.6. Final Detailing (Fillets)

Always apply fillets last.

- Flange Neck: Large radius (R10) to reduce stress concentration.

- Thread-Spline Step: Small radius (R3-R5) at the base of the vertical step. This is critical to prevent cracking.

## 6. Manufacturing & Assembly Considerations

### 6.1 Manufacturing (Machining)

- Turning: The main profile (Thread, Taper, Shaft) is turned on a CNC Lathe.

- Spline Cutting: The tapered splines are cut using a 5-axis milling machine or a specialized hobbing cutter. The cutter path follows the taper angle calculated in our variable table.

- Drilling: Flange holes are drilled and chamfered.

### 6.2 Assembly Logic

- Propeller Installation: The propeller slides over the Thread start cross section (Clearance ensures no contact).

- Spline Engagement: The propeller teeth pass through the gaps in the thread-spline connected cross section step and engage the shaft splines.

- Taper Lock: The propeller slides up the taper until it creates a friction fit.

- Locking: A Cone Nut is tightened onto the Thread start cross section, pushing the propeller firmly onto the taper.

### 6.3 Corrosion Protection

Since the threaded end protrudes into seawater:

- Material: Duplex 2205 resists pitting.

- Cap Nut: A watertight cone nut covers the threads.

- Anode: A Zinc anode is attached to the nut to provide cathodic protection.

## 7. Conclusion

This design represents a robust, modern approach to marine shaft engineering. By combining Hollow Geometry for weight reduction with Tapered Splines for fatigue resistance, it outperforms traditional solid, keyed shafts. The Parametric Model ensures that the design is flexible and future-proof, ready for immediate FEA simulation in COMSOL.

## 8. Appendix: The Master Variable Table

Use these exact formulas to replicate the parametric logic.

| Type | Name | Value | Units | Formula | Exposed Name |
| --- | --- | --- | --- | --- | --- |
| Dim | R_Thread | 47.00 | mm | = R_SmallEnd - Spline_Depth - Thread_TaperSpline_Clearence | Thread Radius |
| Dim | R_SmallEnd | 53.00 | mm |  | Taper SmallEnd Radius |
| Dim | L_Taper | 300.00 | mm |  | Taper Length |
| Dim | T_Flange | 60.00 | mm |  | Flange Thickness |
| Dim | L_Hollow | 1040.00 | mm | = L_Total - T_Flange - L_Solid_Thread_Taper_Length | Hollow Length |
| Dim | IR_Shaft | 30.00 | mm |  | Shaft Inner Radius |
| Dim | OR_Shaft | 62.50 | mm |  | Shaft Outer Radius |
| Dim | OR_Flange | 125.00 | mm |  | Flange Outer Radius |
| Dim | L_Solid_Thread_Taper_Length | 400.00 | mm | = L_Thread + L_Taper | Solid thread + Taper Section Length |
| Dim | Spline_Half_Angle_Smallend | 15.00 | deg | = Spline_Angle_Smallend / 2 | Spline Half Angle Smallend |
| Dim | Spline_Angle_Smallend | 30.00 | deg | = 360 / ( No_Spline + No_Space_Between_Spline ) | Spline Angle Smallend |
| Dim | OD_Spline_Smallend | 106.00 | mm | = R_SmallEnd * 2 | Spline Outer Diameter Smallend |
| Dim | ID_Spline_Smallend | 96.00 | mm | = ( R_SmallEnd - Spline_Depth ) * 2 | Spline Inner Diameter Smallend |
| Dim | Width_Bearing1 | 67.50 | mm | = Width_Bearing | Bearing1 Width |
| Dim | Width_Half_Bearing1 | 33.75 | mm | = Width_Bearing / 2 | Bearing1 Half Width |
| Dim | Width_Bearing2 | 67.50 | mm | = Width_Bearing | Bearing2 Width |
| Dim | Width_Half_Bearing2 | 33.75 | mm | = Width_Bearing / 2 | Bearing2 Half Width |
| Dim | L_Bearing2_From_Bearing1 | 597.16 | mm |  | Length Of Bearing2 From Bearing1 |
| Dim | L_Bearing1_From_Spline | 230.15 | mm |  | Length Of Bearing1 From Spline |
| Dim | Spline_Half_Angle_Biggerend | 15.00 | deg | = Spline_Half_Angle_Smallend | Spline Half Angle Biggerend |
| Dim | Spline_Angle_Biggerend | 30.00 | deg | = Spline_Angle_Smallend | Spline Angle Biggerend |
| Dim | ID_Spline_Biggerend | 115.00 | mm | = ( OR_Shaft - Spline_Depth ) * 2 | Spline Inner Diameter Biggerend |
| Dim | OD_Spline_Biggerend | 125.00 | mm | = OR_Shaft * 2 | Spline Outer Diameter Biggerend |
| Dim | Shaft_Hole_Fillet_Radius | 5.00 | mm |  | Shaft Hole Fillet Radius |
| Dim | Spline_To_Shaft_Fillet_Radius | 5.00 | mm |  | Spline To Shaft Fillet Radius |
| Dim | Spline_Cut_Smallend_Sketch_Line1 | 72.50 | mm | = OR_Shaft + 10 | Spline Cut Smallend Sketch Line1 |
| Dim | Spline_Cut_Smallend_Sketch_Line2 | 72.50 | mm | = OR_Shaft + 10 | Spline Cut Smallend Sketch Line2 |
| Dim | Spline_Cut_Biggerend_Sketch_Line1 | 72.50 | mm | = OR_Shaft + 10 | Spline Cut Biggerend Sketch Line1 |
| Dim | Spline_Cut_Biggerend_Sketch_Line2 | 72.50 | mm | = OR_Shaft + 10 | Spline Cut Biggerend Sketch Line2 |
| Dim | L_Thread | 100.00 | mm |  | Thread Length |
| Dim | Bearing_Surface_Seperating_Planes_FiniteDepth | 135.00 | mm | = OR_Shaft * 2 + 10 | Bearing Surface Seperating Planes FiniteDepth |
| Dim | Flange_PCD | 200.00 | mm | = OR_Flange * 2 - ( Flange_Hole_Dia / 2 ) - 40 | Pitch Circle Diameter |
| Dim | L_Bearing2_Edge2_Plane | 135.00 | mm | = OR_Shaft * 2 + 10 | Length Of Bearing2 Edge2 Plane |
| Dim | L_Bearing2_Edge1_Plane | 135.00 | mm | = OR_Shaft * 2 + 10 | Length Of Bearing2 Edge1 Plane |
| Dim | L_Bearing1_Edge2_Plane | 135.00 | mm | = OR_Shaft * 2 + 10 | Length Of Bearing1 Edge2 Plane |
| Dim | L_Bearing1_Edge1_Plane | 135.00 | mm | = OR_Shaft * 2 + 10 | Length Of Bearing1 Edge1 Plane |
| Dim | Thread_To_Spline_Fillet_Radius | 3.00 | mm |  | Thread To Spline Fillet Radius |
| Dim | Shaft_To_Splange_Fillet_Radius | 10.00 | mm |  | Shaft To Splange Fillet Radius |
| Dim | Flange_Hole_Dia | 20.00 | mm |  | Bolt Holes Diameter |
| Var | PhysicalProperties_Density | 0.000 | kg/m^3 |  | Density |
| Var | PhysicalProperties_Accuracy | 0.990 |  |  | Accuracy |
| Var | L_Total | 1500.00 | mm |  | Shaft Total Length |
| Var | Spline_Depth | 5.00 | mm |  | Spline Depth |
| Var | No_Space_Between_Spline | 6.000 |  | = No_Spline | Number of Spaces Between Spline |
| Var | Shaft_Taper_Angle | 1.81 | deg | = atn(( OR_Shaft - R_SmallEnd ) / L_Taper ) * ( 180 / 3.14159 ) | Shaft Taper Angle |
| Var | Width_Bearing | 67.50 | mm | = OR_Shaft + Width_Bearing_Tolerence | Bearing Width |
| Var | Width_Bearing_Tolerence | 5.00 | mm |  | Bearing Width Tolerence |
| Var | No_Spline | 6.000 |  |  | Number Of Spline |
| Var | Thread_TaperSpline_Clearence | 1.00 | mm |  | Thread Taper And Spline Clearence |
| Var | No_Flange_Bolts | 6.000 |  |  | Number Of Bolts In Flange |